ISO Collaboration
Prepared 2017-03-07, revised 2017-03-11 by Bill Claff

### Introduction

The purpose of the collaboration is measure where neutral gray appears in the linear data relative to the nominal 18% that we might expect.

DxOMark calls this "Measured ISO" and  determines it in a very rigorous scientific way.

The approach taken here is less scientific but is highly repeatable and provides real-world information.

### Preparing your Camera to take the Images

Choose a lightweight prime lens. Not extremely wide as we want to avoid a large light falloff, 
For example, I used a 30mm f/1.4 to collect my data.
If your camera has a fixed lens your choices are more limited; if it is a zoom lens don't use the widest focal length.

Use RAW or RAW+JPG. Only the RAW files are used by the analysis.
For cameras like Fujifilm set Dynamic Range to 100% not Auto, 200% or 400%.
For Nikon cameras it's probably best to set Active D-Lighting (ADL) to Off.

You will be using Aperture mode at f/4, Manual focus focused at infinity at ISO 200.
The test is sensitive to metering mode but unfortunately the best mode seems to vary.
Center-weighted metering is probably ideal but Matrix metering seems to work OK with most camera, for example Nikons.
When in doubt just use your typical one.

### Gray Patch Tool

We're going to take at least 18 images of various gray patches on the computer screen.

Having the camera a consistent distance from the screen is very helpful.
I simply put the filter ring of my lens directly against my computer screen.
You may want to use a tripod, but hand-held is OK.

Use the Gray Patch Tool.

This puts a window on the screen with an RGB value of (0, 0, 0).
The values are shown in hexadecimal. So at this point you will see 000000 @ 0F.
The ‘a’ or ‘-‘ keys will reduce the gray value by 0F on each key press.
The ‘z’ or ‘+’ keys will increase the gray value by 0F on each key press.

Enlarge or maximize the window.

Take images, pressing the ‘z’ or ‘+‘ key between each image.
At the default step size there will be 18 images from 000000 to FFFFFF.
Exposure time will start at about 5s and drop to about 1/30s; depending on your screen brightness.
If you feel ambitious you can use the 'd' key to decrease the step size and increase the number of images.