Analysis of Focal Lengths 18-200mm versus 18-70mm
Prepared 2006-01-09 (146/47202) by Bill Claff

I measured the "true" focal lengths of these lenses at each marked focal length on the lenses.
(OK, actually I missed 24mm on the 18-70mm and measure 60mm instead)
(I used the technique outlined in Measuring Focal Length)

Here is a table:

|  | 18-200mm |  | 18-70mm |  |
| --- | --- | --- | --- | --- |
|  | infinity | nearest | infinity | nearest |
| 18mm | 18.8 | 17.8 | 18.2 | 16.7 |
| 24mm | 24.2 | 22.8 |  |  |
| 35mm | 35.0 | 31.8 | 32.3 | 27.5 |
| 50mm | 51.8 | 43.9 | 52.8 | 39.4 |
| 70mm | 72.4 | 56.9 | 70.8 | 45.8 |
| 135mm | 147.5 | 81.7 |  |  |
| 200mm | 215.0 | 87.9 |  |  |

And a graph:


summarizing my findings.

I know the values for the 18-200mm for 135mm and 200mm look high, but I double-checked my findings.
(This involves a linear fit of several data-points and the fit is quite good.)
Perhaps it's a sample variation as I also found that I could get a 1:4.09 reproduction ratio and the spec says 1:4.5

Note that despite concerns that the 18-200mm loses so much focal length during close focusing that in fact the 18-70mm loses even more.
I have confirmed, as the chart implies; that a shot at 70mm with the 18-70mm lens has about the same Field Of View as a shot at 50mm with the 18-200mm.