Angle Of View and Focal Length
Prepared 2005-11-11 by Bill Claff

### Introduction

In terms of lens specifications angle of view and focal length are measured strictly at infinity focus.
Angle of view at finite focus distances is meaningful but that is a separate topic and not covered here.

Angle of view is typically reported as a diagonal picture angle for a particular film/sensor size.
Sometimes the film/sensor size used is not stated or obvious.

Diagonal picture angle at a particular diagonal size is related to focal length by a simple trigonometric formula.
diagonal picture angle=2∗arctandiagonal size2focal length

You can also rearrange this formula to obtain focal length from diagonal picture angle and diagonal size.
focal length=diagonal size2∗tandiagonal picture angle2

Some manufacturers, Nikon in particular; provide a more precise diagonal picture angle than focal length in their lens specification.
Stated focal lengths are typically rounded to the nearest mm; sometimes further.
So if diagonal picture angle is more precise then a more precise focal length can be computed using the focal length formula.

Nikon reports diagonal picture angle with a precision of 10 minutes or 1/6th degree.
By adding and subtracting ½ the precision from the reported diagonal picture angle we can compute a range of focal lengths; the “true” focal length falls in this range and the range implies focal length accuracy.

The following table lists picture angle and focal length data for a number of lenses.

The data in the green cells is entered specifications.
The data in the light green cells is the desired film/sensor size for picture angle calculations.
The data in the yellow cells is computed.

Zoom lenses are listed twice, once at each focal length.
Bold lens descriptions are auto focus, non-bold descriptions are manual focus.

The computed horizontal pictures angle and vertical pictures angle are primarily of interest to photographers doing panoramic photography for the purpose of performing corrections before stitching images.
For example, I use 88.4 and 66.0 doing panoramic stitching of images taken with my 12-24mm f/4G ED AF-S DX Zoom-Nikkor lens.

The computed focal length is generally within the accuracy of the published focal length; exceptions are listed in the column labeled as corrected.
A common example is the 50mm f/1.8D AF Nikkor; the computed focal length is 51mm rather than 50mm.

It appears from this table that only Nikon states picture angles that are independent from and not simply computed from the focal length.

### Table (Excel workbook)

| sensor or film |  | picture angle |  |  |  | focal length |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | computed |  | published |  |  | computed |  |  |  |  |
| horizontal | vertical | horizontal | vertical |  |  |  |  | accuracy | corrected | Lens |  |
| mm | mm | degrees | degrees | degrees | minutes | mm | mm | mm | mm |  |  |
|  |  | at | at |  |  |  |  |  |  |  |  |
|  |  | 23.4 | 15.6 |  |  |  |  |  |  |  |  |
|  |  | mm | mm |  |  |  |  |  |  |  |  |
| 23.4 | 15.6 | 88.42 | 65.93 | 99 | 0 | 12 | 12.01 | 0.02 |  | Nikon | 12-24mm f/4G ED-IF AF-S DX Zoom-Nikkor |
| 36 | 24 | 79.49 | 58.00 | 114 | 0 | 14 | 14.05 | 0.02 |  | Nikon | 14mm f/2.8D ED AF Nikkor |
| 36 | 24 | 69.30 | 49.48 | 104 | 0 | 17 | 16.90 | 0.03 |  | Nikon | 17-35mm f/2.8D ED-IF AF-S Zoom-Nikkor |
| 23.4 | 15.6 | 68.81 | 49.08 | 79 | 0 | 17 | 17.06 | 0.03 |  | Nikon | 17-55mm f/2.8G ED-IF AF-S DX Zoom-Nikkor |
| 23.4 | 15.6 | 65.97 | 46.80 | 76 | 0 | 18 | 18.00 | 0.03 |  | Nikon | 18-200mm f/3.5-5.6 ED-IF AF-S VR DX Zoom-Nikkor |
| 23.4 | 15.6 | 65.97 | 46.80 | 76 | 0 | 18 | 18.00 | 0.03 |  | Nikon | 18-55mm f/3.5-5.6G ED AF-S DX Zoom-Nikkor |
| 23.4 | 15.6 | 65.97 | 46.80 | 76 | 0 | 18 | 18.00 | 0.03 |  | Nikon | 18-70mm f3.5-4.5G ED-IF AF-S DX Zoom Nikkor |
| 36 | 24 | 65.53 | 46.44 | 100 | 0 | 18 | 18.15 | 0.03 |  | Nikon | 18-35mm f/3.5-4.5D ED-IF AF Zoom-Nikkor |
| 36 | 24 | 65.53 | 46.44 | 100 | 0 | 18 | 18.15 | 0.03 |  | Nikon | 18mm f/2.8D AF Nikkor |
| 36 | 24 | 60.15 | 42.22 | 94 | 0 | 20 | 20.17 | 0.03 |  | Nikon | 20mm f/2.8D AF Nikkor |
| 36 | 24 | 52.26 | 36.22 | 84 | 30 | 24 | 23.82 | 0.03 |  | Nikon | 24-85mm f/2.8-4D IF AF Zoom-Nikkor |
| 36 | 24 | 52.26 | 36.22 | 84 | 30 | 24 | 23.82 | 0.03 |  | Nikon | 24-85mm f/3.5-4.5G ED-IF AF-S Zoom-Nikkor |
| 23.4 | 15.6 | 52.15 | 36.13 | 61 | 0 | 24 | 23.87 | 0.04 |  | Nikon | 12-24mm f/4G ED-IF AF-S DX Zoom-Nikkor |
| 36 | 24 | 51.86 | 35.92 | 84 | 0 | 24 | 24.03 | 0.04 |  | Nikon | 24-120mm f/3.5-5.6G ED-IF AF-S VR Zoom-Nikkor |
| 36 | 24 | 51.86 | 35.92 | 84 | 0 | 24 | 24.03 | 0.04 |  | Nikon | 24mm f/2 Nikkor AI-S |
| 36 | 24 | 51.86 | 35.92 | 84 | 0 | 24 | 24.03 | 0.04 |  | Nikon | 24mm f/2.8D AF Nikkor |
| 36 | 24 | 44.53 | 30.53 | 74 | 20 | 28 | 28.54 | 0.04 |  | Nikon | 28-100mm f/3.5-5.6G Zoom-Nikkor |
| 36 | 24 | 44.53 | 30.53 | 74 | 20 | 28 | 28.54 | 0.04 |  | Nikon | 28-105mm f/3.5-4.5D AF Zoom-Nikkor |
| 36 | 24 | 44.53 | 30.53 | 74 | 20 | 28 | 28.54 | 0.04 |  | Nikon | 28-200mm f/3.5-5.6G ED-IF AF Zoom-Nikkor |
| 36 | 24 | 44.53 | 30.53 | 74 | 20 | 28 | 28.54 | 0.04 |  | Nikon | 28-70mm f/2.8 ED-IF AF-S Zoom-Nikkor |
| 36 | 24 | 44.29 | 30.36 | 74 | 0 | 28 | 28.71 | 0.04 | 29 | Nikon | 28-80mm f/3.3-5.6G AF Zoom-Nikkor |
| 36 | 24 | 44.29 | 30.36 | 74 | 0 | 28 | 28.71 | 0.04 | 29 | Nikon | 28mm f/1.4D AF Nikkor |
| 36 | 24 | 44.29 | 30.36 | 74 | 0 | 28 | 28.71 | 0.04 | 29 | Nikon | 28mm f/2 Nikkor AI-S |
| 36 | 24 | 44.29 | 30.36 | 74 | 0 | 28 | 28.71 | 0.04 | 29 | Nikon | 28mm f/2.8D AF Nikkor |
| 36 | 24 | 44.29 | 30.36 | 74 | 0 | 28 | 28.71 | 0.04 | 29 | Nikon | 28mm f/3.5 PC Nikkor AI-S |
| 36 | 24 | 36.17 | 24.57 | 62 | 20 | 35 | 35.77 | 0.06 | 36 | Nikon | 35-70mm f/2.8D AF Zoom-Nikkor |
| 36 | 24 | 35.95 | 24.41 | 62 | 0 | 35 | 36.00 | 0.06 | 36 | Nikon | 17-35mm f/2.8D ED-IF AF-S Zoom-Nikkor |
| 36 | 24 | 35.95 | 24.41 | 62 | 0 | 35 | 36.00 | 0.06 | 36 | Nikon | 18-35mm f/3.5-4.5D ED-IF AF Zoom-Nikkor |
| 36 | 24 | 35.95 | 24.41 | 62 | 0 | 35 | 36.00 | 0.06 | 36 | Nikon | 35-200mm f/3.5-4.5 Zoom-Nikkor AI-S |
| 36 | 24 | 35.95 | 24.41 | 62 | 0 | 35 | 36.00 | 0.06 | 36 | Nikon | 35mm f/1.4 Nikkor AI-S |
| 36 | 24 | 35.95 | 24.41 | 62 | 0 | 35 | 36.00 | 0.06 | 36 | Nikon | 35mm f/2D AF Nikkor |
| 36 | 24 | 28.26 | 19.05 | 50 | 0 | 45 | 46.39 | 0.09 | 46 | Nikon | 45mm f/2.8P Nikkor AI-S |
| 36 | 24 | 25.81 | 17.37 | 46 | 0 | 50 | 50.96 | 0.10 | 51 | Nikon | 50mm f/1.2 Nikkor AI-S |
| 36 | 24 | 25.81 | 17.37 | 46 | 0 | 50 | 50.96 | 0.10 | 51 | Nikon | 50mm f/1.4D AF Nikkor |
| 36 | 24 | 25.81 | 17.37 | 46 | 0 | 50 | 50.96 | 0.10 | 51 | Nikon | 50mm f/1.8D AF Nikkor |
| 23.4 | 15.6 | 24.08 | 16.18 | 28 | 50 | 55 | 54.70 | 0.17 |  | Nikon | 17-55mm f/2.8G ED-IF AF-S DX Zoom-Nikkor |
| 23.4 | 15.6 | 24.08 | 16.18 | 28 | 50 | 55 | 54.70 | 0.17 |  | Nikon | 18-55mm f/3.5-5.6G ED AF-S DX Zoom-Nikkor |
| 23.4 | 15.6 | 24.08 | 16.18 | 28 | 50 | 55 | 54.70 | 0.17 |  | Nikon | 55-200mm f/4-5.6G ED AF-S DX Zoom-Nikkor |
| 36 | 24 | 22.03 | 14.79 | 39 | 40 | 60 | 59.98 | 0.14 |  | Nikon | 60mm f/2.8D AF Micro-Nikkor |
| 23.4 | 15.6 | 19.01 | 12.74 | 22 | 50 | 70 | 69.63 | 0.26 |  | Nikon | 18-70mm f3.5-4.5G ED-IF AF-S DX Zoom Nikkor |
| 36 | 24 | 18.92 | 12.68 | 34 | 20 | 70 | 70.03 | 0.18 |  | Nikon | 28-70mm f/2.8 ED-IF AF-S Zoom-Nikkor |
| 36 | 24 | 18.92 | 12.68 | 34 | 20 | 70 | 70.03 | 0.18 |  | Nikon | 35-70mm f/2.8D AF Zoom-Nikkor |
| 36 | 24 | 18.92 | 12.68 | 34 | 20 | 70 | 70.03 | 0.18 |  | Nikon | 70-200mm f/2.8G ED-IF AF-S VR Zoom-Nikkor |
| 36 | 24 | 18.92 | 12.68 | 34 | 20 | 70 | 70.03 | 0.18 |  | Nikon | 70-300mm f/4-5.6D ED AF Zoom-Nikkor |
| 36 | 24 | 18.92 | 12.68 | 34 | 20 | 70 | 70.03 | 0.18 |  | Nikon | 70-300mm f/4-5.6G AF Zoom-Nikkor |
| 36 | 24 | 16.54 | 11.07 | 30 | 10 | 80 | 80.27 | 0.23 |  | Nikon | 28-80mm f/3.3-5.6G AF Zoom-Nikkor |
| 36 | 24 | 16.54 | 11.07 | 30 | 10 | 80 | 80.27 | 0.23 |  | Nikon | 80-200mm f/2.8D ED AF Zoom-Nikkor |
| 36 | 24 | 16.54 | 11.07 | 30 | 10 | 80 | 80.27 | 0.23 |  | Nikon | 80-400mm f/4.5-5.6D ED VR AF Zoom-Nikkor |
| 36 | 24 | 15.59 | 10.43 | 28 | 30 | 85 | 85.18 | 0.26 |  | Nikon | 24-85mm f/2.8-4D IF AF Zoom-Nikkor |
| 36 | 24 | 15.59 | 10.43 | 28 | 30 | 85 | 85.18 | 0.26 |  | Nikon | 24-85mm f/3.5-4.5G ED-IF AF-S Zoom-Nikkor |
| 36 | 24 | 15.59 | 10.43 | 28 | 30 | 85 | 85.18 | 0.26 |  | Nikon | 85mm f/1.4 Nikkor AI-S |
| 36 | 24 | 15.59 | 10.43 | 28 | 30 | 85 | 85.18 | 0.26 |  | Nikon | 85mm f/1.4D AF Nikkor |
| 36 | 24 | 15.59 | 10.43 | 28 | 30 | 85 | 85.18 | 0.26 |  | Nikon | 85mm f/2.8 PC Micro-Nikkor |
| 36 | 24 | 13.26 | 8.86 | 24 | 20 | 100 | 100.34 | 0.36 |  | Nikon | 28-100mm f/3.5-5.6G Zoom-Nikkor |
| 36 | 24 | 12.79 | 8.55 | 23 | 30 | 105 | 104.01 | 0.38 | 104 | Nikon | 105mm f/2D AF DC-Nikkor |
| 36 | 24 | 12.70 | 8.48 | 23 | 20 | 105 | 104.77 | 0.39 |  | Nikon | 105mm f/2.5 Nikkor AI-S |
| 36 | 24 | 12.70 | 8.48 | 23 | 20 | 105 | 104.77 | 0.39 |  | Nikon | 105mm f/2.8D AF Micro-Nikkor |
| 36 | 24 | 12.70 | 8.48 | 23 | 20 | 105 | 104.77 | 0.39 |  | Nikon | 28-105mm f/3.5-4.5D AF Zoom-Nikkor |
| 36 | 24 | 11.13 | 7.43 | 20 | 30 | 120 | 119.63 | 0.50 |  | Nikon | 24-120mm f/3.5-5.6G ED-IF AF-S VR Zoom-Nikkor |
| 36 | 24 | 9.75 | 6.51 | 18 | 0 | 135 | 136.59 | 0.65 | 137 | Nikon | 135mm f/2.8 Nikkor AI-S |
| 36 | 24 | 9.75 | 6.51 | 18 | 0 | 135 | 136.59 | 0.65 | 137 | Nikon | 135mm f/2D AF DC-Nikkor |
| 36 | 24 | 7.37 | 4.92 | 13 | 40 | 180 | 180.53 | 1.12 |  | Nikon | 180mm f/2.8D ED-IF AF Nikkor |
| 36 | 24 | 6.64 | 4.43 | 12 | 20 | 200 | 200.22 | 1.37 |  | Nikon | 200-400mm f/4G ED-IF AF-S VR Zoom-Nikkor |
| 36 | 24 | 6.64 | 4.43 | 12 | 20 | 200 | 200.22 | 1.37 |  | Nikon | 200mm f/2G ED-IF AF-S VR Nikkor |
| 36 | 24 | 6.64 | 4.43 | 12 | 20 | 200 | 200.22 | 1.37 |  | Nikon | 200mm f/4 Micro-Nikkor IF AI-S |
| 36 | 24 | 6.64 | 4.43 | 12 | 20 | 200 | 200.22 | 1.37 |  | Nikon | 200mm f/4D ED-IF AF Micro-Nikkor |
| 36 | 24 | 6.64 | 4.43 | 12 | 20 | 200 | 200.22 | 1.37 |  | Nikon | 28-200mm f/3.5-5.6G ED-IF AF Zoom-Nikkor |
| 36 | 24 | 6.64 | 4.43 | 12 | 20 | 200 | 200.22 | 1.37 |  | Nikon | 35-200mm f/3.5-4.5 Zoom-Nikkor AI-S |
| 36 | 24 | 6.64 | 4.43 | 12 | 20 | 200 | 200.22 | 1.37 |  | Nikon | 70-200mm f/2.8G ED-IF AF-S VR Zoom-Nikkor |
| 36 | 24 | 6.64 | 4.43 | 12 | 20 | 200 | 200.22 | 1.37 |  | Nikon | 80-200mm f/2.8D ED AF Zoom-Nikkor |
| 23.4 | 15.6 | 6.59 | 4.40 | 8 | 0 | 200 | 201.09 | 2.12 |  | Nikon | 18-200mm f/3.5-5.6 ED-IF AF-S VR DX Zoom-Nikkor |
| 23.4 | 15.6 | 6.59 | 4.40 | 8 | 0 | 200 | 201.09 | 2.12 |  | Nikon | 55-200mm f/4-5.6G ED AF-S DX Zoom-Nikkor |
| 36 | 24 | 4.38 | 2.92 | 8 | 10 | 300 | 303.04 | 3.13 |  | Nikon | 300mm f/2.8D ED-IF II AF-S |
| 36 | 24 | 4.38 | 2.92 | 8 | 10 | 300 | 303.04 | 3.13 |  | Nikon | 300mm f/2.8G ED-IF AF-S VR Nikkor |
| 36 | 24 | 4.38 | 2.92 | 8 | 10 | 300 | 303.04 | 3.13 |  | Nikon | 300mm f/4D ED-IF AF-S Nikkor |
| 36 | 24 | 4.38 | 2.92 | 8 | 10 | 300 | 303.04 | 3.13 |  | Nikon | 70-300mm f/4-5.6D ED AF Zoom-Nikkor |
| 36 | 24 | 4.38 | 2.92 | 8 | 10 | 300 | 303.04 | 3.13 |  | Nikon | 70-300mm f/4-5.6G AF Zoom-Nikkor |
| 36 | 24 | 3.29 | 2.20 | 6 | 10 | 400 | 401.61 | 5.51 |  | Nikon | 200-400mm f/4G ED-IF AF-S VR Zoom-Nikkor |
| 36 | 24 | 3.29 | 2.20 | 6 | 10 | 400 | 401.61 | 5.51 |  | Nikon | 400mm f/2.8D ED-IF AF-S II Nikkor |
| 36 | 24 | 3.29 | 2.20 | 6 | 10 | 400 | 401.61 | 5.51 |  | Nikon | 80-400mm f/4.5-5.6D ED VR AF Zoom-Nikkor |
| 36 | 24 | 2.66 | 1.77 | 5 | 0 | 500 | 495.48 | 8.41 |  | Nikon | 500mm f/4D ED-IF AF-S II Nikkor |
| 36 | 24 | 2.66 | 1.77 | 5 | 0 | 500 | 495.48 | 8.41 |  | Nikon | 500mm f/8N Reflex Nikkor |
| 36 | 24 | 2.21 | 1.47 | 4 | 10 | 600 | 594.70 | 12.15 |  | Nikon | 600mm f/4D ED-IF AF-S II Nikkor |
| 20.68 | 13.79 | 98.94 | 75.89 | 102.4 |  | 10 | 9.99 | 0.01 |  | Sigma | 10-20mm F4-5.6 EX DC HSM |
| 36 | 24 | 88.53 | 66.03 | 122.0 |  | 12 | 11.99 | 0.01 |  | Sigma | 12-24mm F4-5.6 EX DG ASPHERICAL/ HSM |
| 36 | 24 | 79.74 | 58.22 | 114.2 |  | 14 | 14.00 | 0.01 |  | Sigma | 14mm F2.8 EX Aspherical / Asperical HSM |
| 36 | 24 | 75.31 | 54.45 | 110.0 |  | 15 | 15.15 | 0.01 |  | Sigma | 15-30mm F3.5-4.5 EX Aspherical DG DF |
| 36 | 24 | 69.05 | 49.27 | 103.7 |  | 17 | 16.99 | 0.02 |  | Sigma | 17-35mmF2.8-4 EX DG Aspherical |
| 20.68 | 13.79 | 66.04 | 46.86 | 69.3 |  | 18 | 17.98 | 0.02 |  | Sigma | 18-125mm F3.5-5.6 DC |
| 20.68 | 13.79 | 66.04 | 46.86 | 69.3 |  | 18 | 17.98 | 0.02 |  | Sigma | 18-200mm F3.5-6.3 DC |
| 20.68 | 13.79 | 66.04 | 46.86 | 69.3 |  | 18 | 17.98 | 0.02 |  | Sigma | 18-50mm F2.8 EX DC |
| 20.68 | 13.79 | 66.04 | 46.86 | 69.3 |  | 18 | 17.98 | 0.02 |  | Sigma | 18-50mm F3.5-5.6 DC |
| 20.68 | 13.79 | 60.68 | 42.63 | 63.8 |  | 20 | 19.97 | 0.02 |  | Sigma | 10-20mm F4-5.6 EX DC HSM |
| 36 | 24 | 60.62 | 42.58 | 94.5 |  | 20 | 20.00 | 0.02 |  | Sigma | 20mm F1.8 EX Aspherical DG DF RF |
| 36 | 24 | 51.97 | 36.00 | 84.1 |  | 24 | 23.98 | 0.02 |  | Sigma | 12-24mm F4-5.6 EX DG ASPHERICAL/ HSM |
| 36 | 24 | 51.97 | 36.00 | 84.1 |  | 24 | 23.98 | 0.02 |  | Sigma | 24-135mm f2.8-4.5 Aspherical IF |
| 36 | 24 | 51.97 | 36.00 | 84.1 |  | 24 | 23.98 | 0.02 |  | Sigma | 24-60mm F2.8 EX DG |
| 36 | 24 | 51.97 | 36.00 | 84.1 |  | 24 | 23.98 | 0.02 |  | Sigma | 24-70mm F2.8 EX DG Macro |
| 36 | 24 | 51.97 | 36.00 | 84.1 |  | 24 | 23.98 | 0.02 |  | Sigma | 24-70mm F3.5-5.6 Aspherical HF |
| 36 | 24 | 51.97 | 36.00 | 84.1 |  | 24 | 23.98 | 0.02 |  | Sigma | 24mm F1.8 EX Aspherical DG DF MACRO |
| 36 | 24 | 45.41 | 31.17 | 75.5 |  | 28 | 27.94 | 0.03 |  | Sigma | 28-70mm F2.8-4 DG High Speed Zoom |
| 36 | 24 | 45.33 | 31.12 | 75.4 |  | 28 | 27.99 | 0.03 |  | Sigma | 28-105mm F3.8-5.6 UC-III Aspherical IF |
| 36 | 24 | 45.33 | 31.12 | 75.4 |  | 28 | 27.99 | 0.03 |  | Sigma | 28-105mmF2.8-4 DG Aspherical IF |
| 36 | 24 | 45.33 | 31.12 | 75.4 |  | 28 | 27.99 | 0.03 |  | Sigma | 28-135mm F3.8-5.6 Aspherical IF Macro |
| 36 | 24 | 45.33 | 31.12 | 75.4 |  | 28 | 27.99 | 0.03 |  | Sigma | 28-200mm F3.5-5.6 DG Compact Aspherical Hyperzoom Macro |
| 36 | 24 | 45.33 | 31.12 | 75.4 |  | 28 | 27.99 | 0.03 |  | Sigma | 28-300mm F3.5-6.3 DG Macro |
| 36 | 24 | 45.33 | 31.12 | 75.4 |  | 28 | 27.99 | 0.03 |  | Sigma | 28-70mm F2.8 EX DG |
| 36 | 24 | 45.33 | 31.12 | 75.4 |  | 28 | 27.99 | 0.03 |  | Sigma | 28mm F1.8 EX Aspherical DG DF MACRO |
| 36 | 24 | 42.58 | 29.13 | 71.6 |  | 30 | 30.00 | 0.03 |  | Sigma | 15-30mm F3.5-4.5 EX Aspherical DG DF |
| 20.68 | 13.79 | 42.55 | 29.11 | 45.0 |  | 30 | 30.01 | 0.04 |  | Sigma | 30mm F1.4 EX DC HSM |
| 36 | 24 | 36.91 | 25.08 | 63.4 |  | 35 | 35.03 | 0.03 |  | Sigma | 17-35mmF2.8-4 EX DG Aspherical |
| 36 | 24 | 26.31 | 17.72 | 46.8 |  | 50 | 49.99 | 0.06 |  | Sigma | 50-500mmF4-6.3 EX DG APO RF / RF HSM |
| 36 | 24 | 26.31 | 17.72 | 46.8 |  | 50 | 49.99 | 0.06 |  | Sigma | 50mm F2.8 EX DG Macro |
| 20.68 | 13.79 | 26.27 | 17.69 | 27.9 |  | 50 | 50.04 | 0.09 |  | Sigma | 18-50mm F2.8 EX DC |
| 20.68 | 13.79 | 26.27 | 17.69 | 27.9 |  | 50 | 50.04 | 0.09 |  | Sigma | 18-50mm F3.5-5.6 DC |
| 20.68 | 13.79 | 24.00 | 16.13 | 25.5 |  | 55 | 54.93 | 0.11 |  | Sigma | 55-200mm F4-5.6 DC |
| 36 | 24 | 22.01 | 14.77 | 39.6 |  | 60 | 60.09 | 0.08 |  | Sigma | 24-60mm F2.8 EX DG |
| 36 | 24 | 18.98 | 12.72 | 34.4 |  | 70 | 69.89 | 0.11 |  | Sigma | 24-70mm F3.5-5.6 Aspherical HF |
| 36 | 24 | 18.92 | 12.68 | 34.3 |  | 70 | 70.10 | 0.11 |  | Sigma | 24-70mm F2.8 EX DG Macro |
| 36 | 24 | 18.92 | 12.68 | 34.3 |  | 70 | 70.10 | 0.11 |  | Sigma | 28-70mm F2.8 EX DG |
| 36 | 24 | 18.92 | 12.68 | 34.3 |  | 70 | 70.10 | 0.11 |  | Sigma | 28-70mm F2.8-4 DG High Speed Zoom |
| 36 | 24 | 18.92 | 12.68 | 34.3 |  | 70 | 70.10 | 0.11 |  | Sigma | 70-200mm F2.8 EX DG APO IF / APO IF HSM |
| 36 | 24 | 18.92 | 12.68 | 34.3 |  | 70 | 70.10 | 0.11 |  | Sigma | 70-300mm F4-5.6 DG APO Macro |
| 36 | 24 | 18.92 | 12.68 | 34.3 |  | 70 | 70.10 | 0.11 |  | Sigma | 70-300mm F4-5.6 DG Macro |
| 36 | 24 | 16.63 | 11.13 | 30.3 |  | 80 | 79.90 | 0.14 |  | Sigma | 80-400mm f4.5-5.6 EX DG APO OS |
| 36 | 24 | 13.31 | 8.90 | 24.4 |  | 100 | 100.06 | 0.21 |  | Sigma | 100-300mm F4 EX DG APO IF / IF HSM |
| 36 | 24 | 12.70 | 8.48 | 23.3 |  | 105 | 104.92 | 0.23 |  | Sigma | 105mm F2.8 EX DG Macro |
| 36 | 24 | 12.70 | 8.48 | 23.3 |  | 105 | 104.92 | 0.23 |  | Sigma | 28-105mm F3.8-5.6 UC-III Aspherical IF |
| 36 | 24 | 12.70 | 8.48 | 23.3 |  | 105 | 104.92 | 0.23 |  | Sigma | 28-105mmF2.8-4 DG Aspherical IF |
| 36 | 24 | 11.09 | 7.41 | 20.4 |  | 120 | 120.23 | 0.30 |  | Sigma | 120-300mm F2.8 EX DG APO HSM IF |
| 20.68 | 13.79 | 10.69 | 7.14 | 11.4 |  | 125 | 124.53 | 0.55 |  | Sigma | 18-125mm F3.5-5.6 DC |
| 36 | 24 | 9.87 | 6.59 | 18.2 |  | 135 | 135.06 | 0.38 |  | Sigma | 135-400mm F4.5-5.6 DG APO Aspherical |
| 36 | 24 | 9.87 | 6.59 | 18.2 |  | 135 | 135.06 | 0.38 |  | Sigma | 24-135mm f2.8-4.5 Aspherical IF |
| 36 | 24 | 9.87 | 6.59 | 18.2 |  | 135 | 135.06 | 0.38 |  | Sigma | 28-135mm F3.8-5.6 Aspherical IF Macro |
| 36 | 24 | 8.89 | 5.93 | 16.4 |  | 150 | 150.12 | 0.47 |  | Sigma | 150mm F2.8 EX DG APO Macro HSM |
| 36 | 24 | 7.84 | 5.23 | 14.5 |  | 170 | 170.05 | 0.59 |  | Sigma | 170-500mm F5-6.3 DG APO Aspherical |
| 36 | 24 | 7.41 | 4.94 | 13.7 |  | 180 | 180.09 | 0.67 |  | Sigma | 180mm F3.5 EX DG APO Macro IF/ IF HSM |
| 20.68 | 13.79 | 6.64 | 4.43 | 7.1 |  | 200 | 200.35 | 1.42 |  | Sigma | 18-200mm F3.5-6.3 DC |
| 20.68 | 13.79 | 6.64 | 4.43 | 7.1 |  | 200 | 200.35 | 1.42 |  | Sigma | 55-200mm F4-5.6 DC |
| 36 | 24 | 6.64 | 4.43 | 12.3 |  | 200 | 200.77 | 0.83 |  | Sigma | 28-200mm F3.5-5.6 DG Compact Aspherical Hyperzoom Macro |
| 36 | 24 | 6.64 | 4.43 | 12.3 |  | 200 | 200.77 | 0.83 |  | Sigma | 70-200mm F2.8 EX DG APO IF / APO IF HSM |
| 36 | 24 | 4.41 | 2.94 | 8.2 |  | 300 | 301.80 | 1.86 |  | Sigma | 100-300mm F4 EX DG APO IF / IF HSM |
| 36 | 24 | 4.41 | 2.94 | 8.2 |  | 300 | 301.80 | 1.86 |  | Sigma | 120-300mm F2.8 EX DG APO HSM IF |
| 36 | 24 | 4.41 | 2.94 | 8.2 |  | 300 | 301.80 | 1.86 |  | Sigma | 28-300mm F3.5-6.3 DG Macro |
| 36 | 24 | 4.41 | 2.94 | 8.2 |  | 300 | 301.80 | 1.86 |  | Sigma | 300mm F2.8 EX DG APO / APO HSM |
| 36 | 24 | 4.41 | 2.94 | 8.2 |  | 300 | 301.80 | 1.86 |  | Sigma | 70-300mm F4-5.6 DG APO Macro |
| 36 | 24 | 4.41 | 2.94 | 8.2 |  | 300 | 301.80 | 1.86 |  | Sigma | 70-300mm F4-5.6 DG Macro |
| 36 | 24 | 3.33 | 2.22 | 6.2 |  | 400 | 399.45 | 3.25 |  | Sigma | 135-400mm F4.5-5.6 DG APO Aspherical |
| 36 | 24 | 3.33 | 2.22 | 6.2 |  | 400 | 399.45 | 3.25 |  | Sigma | 80-400mm f4.5-5.6 EX DG APO OS |
| 36 | 24 | 2.68 | 1.79 | 5.0 |  | 500 | 495.48 | 5.01 |  | Sigma | 170-500mm F5-6.3 DG APO Aspherical |
| 36 | 24 | 2.68 | 1.79 | 5.0 |  | 500 | 495.48 | 5.01 |  | Sigma | 500mm F4.5 EX DG APO / APO HSM |
| 36 | 24 | 2.68 | 1.79 | 5.0 |  | 500 | 495.48 | 5.01 |  | Sigma | 50-500mmF4-6.3 EX DG APO RF / RF HSM |
| 36 | 24 | 1.65 | 1.10 | 3.1 |  | 800 | 799.48 | 13.11 |  | Sigma | 800mm F5.6 EX DG APO / APO HSM |
| 23.4 | 15.6 | 93.06 | 70.24 | 103 | 29 | 11 | 11.09 | 0.00 |  | Tamron | 11-18mm f/4.5-5.6 Di II LD ASPHERICAL [IF] |
| 36 | 24 | 69.07 | 49.29 | 103 | 41 | 17 | 17.00 | 0.00 |  | Tamron | 17-35mm f/2.8-4 Di LD ASPHERICAL [IF] |
| 23.4 | 15.6 | 65.61 | 46.51 | 75 | 32 | 18 | 18.15 | 0.00 |  | Tamron | 11-18mm f/4.5-5.6 Di II LD ASPHERICAL [IF] |
| 23.4 | 15.6 | 65.61 | 46.51 | 75 | 32 | 18 | 18.15 | 0.00 |  | Tamron | 18-200mm f/3.5-6.3 XR Di II LD ASPHERICAL {IF] Macro |
| 36 | 24 | 63.24 | 44.63 | 97 | 25 | 19 | 19.00 | 0.00 |  | Tamron | 19-35mm f/3.5-4.5 |
| 36 | 24 | 51.96 | 36.00 | 84 | 3 | 24 | 24.01 | 0.00 |  | Tamron | 24-135mm f/3.5-5.6 AD ASPHERICAL [IF} Macro |
| 36 | 24 | 51.96 | 36.00 | 84 | 3 | 24 | 24.01 | 0.00 |  | Tamron | 24mm f/2.5 |
| 36 | 24 | 45.35 | 31.13 | 75 | 23 | 28 | 28.00 | 0.00 |  | Tamron | 28-105mm f/2.8 LD ASPHERICAL [IF] |
| 36 | 24 | 45.35 | 31.13 | 75 | 23 | 28 | 28.00 | 0.00 |  | Tamron | 28-105mm f/2.8 LD SPHERICAL [IF] |
| 36 | 24 | 45.35 | 31.13 | 75 | 23 | 28 | 28.00 | 0.00 |  | Tamron | 28-105mm f/4-5.6 [IF] |
| 36 | 24 | 45.35 | 31.13 | 75 | 23 | 28 | 28.00 | 0.00 |  | Tamron | 28-200 f/3.8-5.6 ASPHERICAL XR [IF] Macro |
| 36 | 24 | 45.35 | 31.13 | 75 | 23 | 28 | 28.00 | 0.00 |  | Tamron | 28-200mm f/3.8-5.6 LD ASPHERICAL [IF] SUPER |
| 36 | 24 | 45.35 | 31.13 | 75 | 23 | 28 | 28.00 | 0.00 |  | Tamron | 28-300 f/3.5-6.3 XR Di ASPHERICAL [IF} Macro |
| 36 | 24 | 45.35 | 31.13 | 75 | 23 | 28 | 28.00 | 0.00 |  | Tamron | 28-70mm f/3.5-4.5 |
| 36 | 24 | 45.35 | 31.13 | 75 | 23 | 28 | 28.00 | 0.00 |  | Tamron | 28-75mm f/2.8 XR Di ASPHERICAL [IF] Macro |
| 36 | 24 | 45.35 | 31.13 | 75 | 23 | 28 | 28.00 | 0.00 |  | Tamron | 28-80mm f/3.5-5.6 ASPHERICAL |
| 36 | 24 | 36.95 | 25.11 | 63 | 25 | 35 | 35.02 | 0.01 |  | Tamron | 17-35mm f/2.8-4 Di LD ASPHERICAL [IF] |
| 36 | 24 | 36.95 | 25.11 | 63 | 25 | 35 | 35.02 | 0.01 |  | Tamron | 19-35mm f/3.5-4.5 |
| 36 | 24 | 18.97 | 12.71 | 34 | 21 | 70 | 69.99 | 0.02 |  | Tamron | 28-70mm f/3.5-4.5 |
| 36 | 24 | 18.97 | 12.71 | 34 | 21 | 70 | 69.99 | 0.02 |  | Tamron | 70-210mm f/3.5 |
| 36 | 24 | 18.97 | 12.71 | 34 | 21 | 70 | 69.99 | 0.02 |  | Tamron | 70-210mm f/4-5.6 |
| 36 | 24 | 18.97 | 12.71 | 34 | 21 | 70 | 69.99 | 0.02 |  | Tamron | 70-300mm f/4-5.6 LD Macro 1:2 |
| 36 | 24 | 18.97 | 12.71 | 34 | 21 | 70 | 69.99 | 0.02 |  | Tamron | 70-300mm f/4-5.6 LD Macro 1:3.9 |
| 36 | 24 | 17.72 | 11.87 | 32 | 10 | 75 | 75.03 | 0.02 |  | Tamron | 28-75mm f/2.8 XR Di ASPHERICAL [IF] Macro |
| 36 | 24 | 16.63 | 11.13 | 30 | 15 | 80 | 80.04 | 0.02 |  | Tamron | 28-80mm f/3.5-5.6 ASPHERICAL |
| 36 | 24 | 16.63 | 11.13 | 30 | 15 | 80 | 80.04 | 0.02 |  | Tamron | 80-210mm f/4.5-5.6 |
| 36 | 24 | 14.81 | 9.90 | 27 | 2 | 90 | 89.99 | 0.03 |  | Tamron | 90mm F/2.8 Di Macro 1:1 |
| 36 | 24 | 14.81 | 9.90 | 27 | 2 | 90 | 89.99 | 0.03 |  | Tamron | 90mm f/2.8 Macro 1:1 |
| 36 | 24 | 12.71 | 8.49 | 23 | 17 | 105 | 105.00 | 0.04 |  | Tamron | 28-105mm f/2.8 LD ASPHERICAL [IF] |
| 36 | 24 | 12.71 | 8.49 | 23 | 17 | 105 | 105.00 | 0.04 |  | Tamron | 28-105mm f/2.8 LD SPHERICAL [IF] |
| 36 | 24 | 12.71 | 8.49 | 23 | 17 | 105 | 105.00 | 0.04 |  | Tamron | 28-105mm f/4-5.6 [IF] |
| 36 | 24 | 9.90 | 6.61 | 18 | 12 | 135 | 135.06 | 0.06 |  | Tamron | 24-135mm f/3.5-5.6 AD ASPHERICAL [IF} Macro |
| 36 | 24 | 7.43 | 4.96 | 13 | 42 | 180 | 180.09 | 0.11 |  | Tamron | 180mm f/3.5 Di LD [IF] Macro 1:1 |
| 36 | 24 | 6.69 | 4.46 | 12 | 21 | 200 | 199.95 | 0.14 |  | Tamron | 200-500mm f/5-6.3 Di LD [IF} |
| 36 | 24 | 6.69 | 4.46 | 12 | 21 | 200 | 199.95 | 0.14 |  | Tamron | 28-200 f/3.8-5.6 ASPHERICAL XR [IF] Macro |
| 36 | 24 | 6.69 | 4.46 | 12 | 21 | 200 | 199.95 | 0.14 |  | Tamron | 28-200mm f/3.8-5.6 LD ASPHERICAL [IF] SUPER |
| 23.4 | 15.6 | 6.64 | 4.43 | 7 | 59 | 200 | 201.51 | 0.21 | 202 | Tamron | 18-200mm f/3.5-6.3 XR Di II LD ASPHERICAL {IF] Macro |
| 36 | 24 | 6.38 | 4.25 | 11 | 46 | 210 | 209.94 | 0.15 |  | Tamron | 70-210mm f/3.5 |
| 36 | 24 | 6.38 | 4.25 | 11 | 46 | 210 | 209.94 | 0.15 |  | Tamron | 70-210mm f/4-5.6 |
| 36 | 24 | 6.38 | 4.25 | 11 | 46 | 210 | 209.94 | 0.15 |  | Tamron | 80-210mm f/4.5-5.6 |
| 36 | 24 | 4.46 | 2.98 | 8 | 15 | 300 | 299.96 | 0.30 |  | Tamron | 28-300 f/3.5-6.3 XR Di ASPHERICAL [IF} Macro |
| 36 | 24 | 4.46 | 2.98 | 8 | 15 | 300 | 299.96 | 0.30 |  | Tamron | 300mm f/2.8 LD [IF] |
| 36 | 24 | 4.46 | 2.98 | 8 | 15 | 300 | 299.96 | 0.30 |  | Tamron | 300mm f/2.8 LD [IF} |
| 36 | 24 | 4.46 | 2.98 | 8 | 15 | 300 | 299.96 | 0.30 |  | Tamron | 70-300mm f/4-5.6 LD Macro 1:2 |
| 36 | 24 | 4.46 | 2.98 | 8 | 15 | 300 | 299.96 | 0.30 |  | Tamron | 70-300mm f/4-5.6 LD Macro 1:3.9 |
| 36 | 24 | 2.67 | 1.78 | 4 | 57 | 500 | 500.50 | 0.85 |  | Tamron | 200-500mm f/5-6.3 Di LD [IF} |
| 36 | 24 | 2.67 | 1.78 | 4 | 57 | 500 | 500.50 | 0.85 |  | Tamron | 500mm f/8 |
| 23.4 | 15.6 | 88.33 | 65.85 | 99 | 0 | 12 | 12.01 | 0.04 |  | Tokina | 12-24mm f/4 AT-X 124 PRO DX |
| 36 | 24 | 64.23 | 45.41 | 98 | 40 | 19 | 18.59 | 0.05 |  | Tokina | 19-35mm f/3.5-4.5 AF 193 |
| 23.4 | 15.6 | 52.07 | 36.08 | 61 | 0 | 24 | 23.87 | 0.08 |  | Tokina | 12-24mm f/4 AT-X 124 PRO DX |
| 36 | 24 | 51.93 | 35.97 | 84 | 10 | 24 | 23.96 | 0.07 |  | Tokina | 24-200mm f/3.5-5.6AT-X 242AF |
| 36 | 24 | 45.20 | 31.02 | 75 | 20 | 28 | 28.02 | 0.08 |  | Tokina | 28-105mm f/3.5-4.8 SZ-X 205 |
| 36 | 24 | 45.20 | 31.02 | 75 | 20 | 28 | 28.02 | 0.08 |  | Tokina | 28-200mm f/3.5-5.3 SZ-X 282 |
| 36 | 24 | 45.20 | 31.02 | 75 | 20 | 28 | 28.02 | 0.08 |  | Tokina | 28-70mm f/2.8 AT-X 287 AF PRO SV |
| 36 | 24 | 45.20 | 31.02 | 75 | 20 | 28 | 28.02 | 0.08 |  | Tokina | 28-70mm f/3.9-4.8 SZ-X 270-II |
| 36 | 24 | 45.20 | 31.02 | 75 | 20 | 28 | 28.02 | 0.08 |  | Tokina | 28-80mm f/2.8 AT-X 280AF PRO |
| 36 | 24 | 45.20 | 31.02 | 75 | 20 | 28 | 28.02 | 0.08 |  | Tokina | 28mm f/2.8 SL 28 |
| 36 | 24 | 45.20 | 31.02 | 75 | 20 | 28 | 28.02 | 0.08 |  | Tokina | 28mm f/2.8 SL 28 |
| 36 | 24 | 36.90 | 25.08 | 63 | 30 | 35 | 34.96 | 0.11 |  | Tokina | 19-35mm f/3.5-4.5 AF 193 |
| 36 | 24 | 18.87 | 12.65 | 34 | 20 | 70 | 70.03 | 0.36 |  | Tokina | 28-70mm f/2.8 AT-X 287 AF PRO SV |
| 36 | 24 | 18.87 | 12.65 | 34 | 20 | 70 | 70.03 | 0.36 |  | Tokina | 28-70mm f/3.9-4.8 SZ-X 270-II |
| 36 | 24 | 17.63 | 11.81 | 32 | 10 | 60 | 75.03 | 0.41 | 75 | Tokina | 60-300mm f/4-5.6SZ-X 630 |
| 36 | 24 | 16.59 | 11.10 | 30 | 20 | 80 | 79.81 | 0.46 |  | Tokina | 28-80mm f/2.8 AT-X 280AF PRO |
| 36 | 24 | 16.59 | 11.10 | 30 | 20 | 80 | 79.81 | 0.46 |  | Tokina | 80-200mm f/2.8 AT-X 828AF PRO |
| 36 | 24 | 16.30 | 10.91 | 29 | 50 | 80 | 81.21 | 0.48 | 81 | Tokina | 80-400mm f/4.5-5.6 AT-X 840AF-II |
| 36 | 24 | 13.30 | 8.89 | 24 | 30 | 100 | 99.64 | 0.70 |  | Tokina | 100mm f/2.8 AT-X 100AF PRO D |
| 36 | 24 | 12.28 | 8.20 | 22 | 40 | 105 | 107.94 | 0.82 | 108 | Tokina | 28-105mm f/3.5-4.8 SZ-X 205 |
| 36 | 24 | 6.60 | 4.40 | 12 | 20 | 200 | 200.22 | 2.76 |  | Tokina | 24-200mm f/3.5-5.6AT-X 242AF |
| 36 | 24 | 6.60 | 4.40 | 12 | 20 | 200 | 200.22 | 2.76 |  | Tokina | 28-200mm f/3.5-5.3 SZ-X 282 |
| 36 | 24 | 6.60 | 4.40 | 12 | 20 | 200 | 200.22 | 2.76 |  | Tokina | 80-200mm f/2.8 AT-X 828AF PRO |
| 36 | 24 | 4.42 | 2.95 | 8 | 20 | 300 | 296.95 | 6.08 |  | Tokina | 300mmf/2.8 AT-X 300AF PRO |
| 36 | 24 | 4.42 | 2.95 | 8 | 20 | 300 | 296.95 | 6.08 |  | Tokina | 60-300mm f/4-5.6SZ-X 630 |
| 36 | 24 | 3.27 | 2.18 | 6 | 13 | 400 | 398.37 | 11.00 |  | Tokina | 80-400mm f/4.5-5.6 AT-X 840AF-II |