D4 Read Noise
Prepared 2012-01-11 by Bill Claff (last revised 2012-03-20)

The D4 Read Noise values were determined from the Optical Black data in NEF files.

Here is a table of measured values (in 14-bit ADUs):

| ISO | noise ADU | Gain electrons/ADU | noise electrons |
| --- | --- | --- | --- |
| 50 | 2.346 | 7.22 | 16.9 |
| 62 | 2.346 | 7.22 | 16.9 |
| 79 | 2.388 | 7.22 | 17.2 |
| 100 | 2.385 | 7.22 | 17.2 |
| 125 | 2.493 | 5.73 | 14.3 |
| 158 | 2.553 | 4.55 | 11.6 |
| 200 | 2.371 | 3.61 | 8.6 |
| 251 | 2.556 | 2.87 | 7.3 |
| 317 | 2.680 | 2.27 | 6.1 |
| 400 | 2.977 | 1.81 | 5.4 |
| 503 | 3.288 | 1.43 | 4.7 |
| 634 | 3.781 | 1.14 | 4.3 |
| 800 | 4.179 | 0.903 | 3.8 |
| 1007 | 5.064 | 0.716 | 3.6 |
| 1269 | 5.810 | 0.569 | 3.3 |
| 1600 | 7.196 | 0.451 | 3.2 |
| 2015 | 8.438 | 0.358 | 3.0 |
| 2539 | 10.176 | 0.284 | 2.9 |
| 3200 | 12.520 | 0.226 | 2.8 |
| 4031 | 15.394 | 0.179 | 2.8 |
| 5079 | 19.592 | 0.142 | 2.8 |
| 6400 | 21.262 | 0.113 | 2.4 |
| 8063 | 26.574 | 0.09 | 2.4 |
| 10159 | 32.467 | 0.071 | 2.3 |
| 12800 | 40.698 | 0.071 | 2.9 |
| 16126 | 26.829 | 0.071 | 1.9 |
| 20318 | 31.771 | 0.071 | 2.3 |
| 25600 | 39.195 | 0.071 | 2.8 |
| 51200 | 38.654 | 0.071 | 2.7 |
| 102400 | 41.072 | 0.071 | 2.9 |
| 204800 | 41.282 | 0.071 | 2.9 |

Note that all electron figures are subject to change pending a better measurement of gain.

The following chart shows these values as a function of ISO:


Shown as a log-log chart for clarity.

The open symbols represent ISO values in the “normal” analog gain range while the closed symbols represent to "Lo" and "Hi" values.